Revised Public Notification Handbook

3rd Revision, March 2023

Tier 2 Public Notification Instructions and Templates

The pages that follow contain instructions and templates for issuing Tier 2 public notification. Along with each template are instructions, including the required method of delivery and suggestions for completing individual sections of the notice. These instructions are designed to supplement Chapter 7, so you may see much of the information repeated here. The following templates are included:

- Chemical or Radiological MCLs – Template 2-1

- Fluoride MCL (CWS only) – Template 2-2

- Stage 1 DBPR Total Trihalomethanes (TTHM) or Haloacetic acids (five) (HAA5) MCL Violation – Template 2-3

- Stage 2 DBPR TTHM or HAA5 MCL Violation – Template 2-3

- SWTR Failure to Filter – Template 2-4

- SWTRs Turbidity Exceedance – Template 2-5

- SWTR Disinfection Treatment – Template 2-6

- LCR Failure to Install Corrosion Control – Template 2-7

- LCR Failure to Install Source Water Treatment – Template 2-8

- LCR Failure to Meet Reporting Requirements for Public Education – Template 2-9

- LCR Failure to Replace Lead Service Lines – Template 2-10

- LT2ESWTR Uncovered Finished Water Reservoir Treatment – Template 2-11

- LT2ESWTR Filtered System Failure to Provide or Install an Additional Level of Treatment – Template 2-12

- LT2ESWTR Unfiltered System Failure to Achieve Required Inactivation Level – Template 2-13

- LT2ESWTR Filtered System Failure to Achieve Required Treatment Credit – Template 2-14

- LT2ESWTR Unfiltered System Failure to Maintain Required Inactivation Level – Template 2-15

- LT2ESWTR Unfiltered System Failure to Install Second Disinfectant – Template 2-16

- Filter Backwash Recycling Rule Treatment Technique Violation – Template 2-17

- RTCR Failure of a Seasonal System to Complete Start-Up Procedures – Template 2-18

- Monitoring Violation Elevated to Tier 2 – Template 2-19

- Failure to Comply with the Conditions of an Exemption – Template 2-20

- GWR Failure to Maintain 4-log Treatment of Viruses – Template 2-21

- GWR Failure to Take Corrective Action Within Required Time Frame – Template 2-22

- RTCR Failure to Perform any Level 1 Assessment or a Level 2 Assessment that is Not Triggered by E. coli MCL violations or the Related Corrective Actions – Template 2-23

- RTCR Failure to Perform a Level 2 Assessment Triggered by an E. coli MCL violation or the Related Corrective Actions – Template 2-24

- Special Notice: LT2ESWTR Failure to Conduct Required Cryptosporidium Monitoring for Any Three Months – Template 2-25

- Special Notice: LT2ESWTR Filtered System Failure to Determine and Report Bin Classification – Template 2-26

- Special Notice: LT2ESWTR Unfiltered System Failure to Calculate and Report Mean Cryptosporidium Level – Template 2-27

Each template also includes the mandatory health effects language from Appendix B to 40 CFR 141 Subpart Q (presented in italics in each notice, with an asterisk on either end). This information must be included as written; with additional violation or situation specific information added in the brackets.

You must also include the following italicized language in all notices, where applicable [40 CFR 141.205(d)]. Use of this language does not relieve you of your obligation to take steps reasonably calculated to notify all persons served:

Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.

Instructions for Chemical or Radiological MCLs – Template 2-1

Template on Reverse

Chemical or radiological maximum contaminant level (MCL) violations require Tier 2 public notification and you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Do not use overly technical terminology when describing treatment methods. Listed below are some steps commonly taken by water systems with chemical or radiological violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are working with [local/state agency] to evaluate the water supply and are researching options to correct the problem. These options may include treating the water to remove [contaminant] or connecting to [water system name]’s water supply.

- We have stopped using the contaminated well. We have increased pumping from other wells, and we are investigating drilling a new well.

- We have increased the frequency that we will test the water for [contaminant].

- We have since taken samples at this location and had them tested. These samples show that we meet the standards.

Repeat Notices

If this is an ongoing violation and/or you fluctuate above and below the MCL, you should give the history behind the violation, including the source of contamination, if known. List the date of the initial detection, as well as how levels have changed over time. If levels are changing as a result of treatment, you can indicate this.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

Chemical or Radiological MCLs Notice – Template 2-1

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Has Levels of [Contaminant] Above Drinking Water Standards

Our water system recently violated a drinking water standard. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

We routinely monitor for the presence of drinking water contaminants. On [give date], we received notice that the sample collected on [give date] showed that our system exceeds the standard, or maximum contaminant level (MCL), for [contaminant]. The standard for [contaminant] is [MCL]. The average level of [contaminant] over the last year has been [provide level]. OR [Contaminant] was found at [provide level].

What should I do?

- There is nothing you need to do. Do not boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours. However, [Insert relevant health effects language for the contaminant from Appendix B to 40 CFR 141 Subpart Q or Appendix B of this document.]

What is being done?

[Describe corrective action.] We anticipate resolving the problem within [estimated time frame].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Fluoride MCL – Template 2-2

Template on Reverse

Since exceeding the fluoride maximum contaminant level (MCL) requires Tier 2 public notification and you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

In addition, CWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language
Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Do not use overly technical terminology when describing treatment methods. Listed below are some steps commonly taken by water systems with fluoride violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are working with [local/state agency] to evaluate the water supply and researching options to correct the problem. These options may include treating the water to remove fluoride or connecting to [water system name]’s water supply.

- We have adjusted the amount of fluoride added to the water so that levels are lower and we meet the standards.

- We have stopped using the contaminated well. We have increased pumping from other wells, and we are investigating drilling a new well.

- We will increase the frequency at which we test the water for fluoride.

- We have since taken samples at this location and had them tested. They show that we meet the standards.

Repeat Notices

If this is an ongoing violation and/or you fluctuate above and below the MCL, you should give the history behind the violation, including the source of contamination, if known. List the date of the initial detection, as well as how levels have changed over time. If levels are changing as a result of treatment, you can indicate this.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

Fluoride MCL Notice – Template 2-2

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Has Levels of Fluoride Above Drinking Water Standards

Our water system recently violated a drinking water standard. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

We routinely monitor for the presence of drinking water contaminants. On [give date], we received notice that the sample collected on [give date] showed that our system exceeds the standard, or maximum contaminant level (MCL), for fluoride. The average level of fluoride in samples taken during the last year was [provide level and units]. The standard for fluoride is that the average of samples taken over the last year may not exceed [MCL].

What should I do?

- Children under the age of nine should use an alternative source of water that is low in fluoride. In addition, you may want to consult your dentist about whether to avoid dental products containing fluoride. Adults and children over age nine should consult their dentist or doctor and show him/her this notice to determine if an alternate source of water low in fluoride should be used.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours. Fluoride in small amounts helps prevent tooth decay. *However, some people who drink water containing fluoride in excess of the MCL over many years could get bone disease, including pain and tenderness of the bones. Fluoride in drinking water at half the MCL or greater may cause mottling of children’s teeth, usually in children less than nine years old. Mottling, also known as dental fluorosis, may include brown staining and/or pitting of the permanent teeth. This problem occurs only in developing teeth, before they erupt from the gums.* Although it takes many years of exposure to fluoride for bone disease to develop, mottling can occur after a relatively short period of exposure.

What is being done?

[Describe corrective action.] We anticipate resolving the problem within [estimated time frame].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for TTHM or HAA5 MCL Violation – Template 2-3

Template on Reverse

If your system’s locational running annual average for Total Trihalomethanes (TTHM) exceeds the maximum contaminant level (MCL) of 0.080 milligrams per liter (mg/L) or the locational running annual average for Haloacetic Acid 5 (HAA5) exceeds the MCL of 0.060 mg/L at one or more sampling locations, you must provide Tier 2 public notification to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below is one step commonly taken by water systems with TTHM or HAA5 MCL violations. You can include the following statement, if appropriate, or develop your own text:

- We are working to minimize the formation of [TTHM or HAA5] while ensuring we maintain an adequate level of disinfectant. We have taken additional steps to change disinfectant type/levels, remove natural organic matter, and increased flushing of water lines to determine if our efforts have been effective.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

TTHM or HAA5 MCL Violation Notice – Template 2-3

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Contaminant, i.e., Total Trihalomethanes (TTHM) or Haloacetic Acids (HAA5)] MCL Violation at [Water System Name]

Our water system recently violated a drinking water standard. Although this incident was not an emergency, as our customers, you have a right to know what happened and what we did (are doing) to correct this situation.

We routinely monitor for the presence of drinking water contaminants. Testing results from [provide date range] show that our system exceeds the standard, or maximum contaminant level (MCL), for [TTHM/HAA5]. The standard for [TTHM/HAA5] is [MCL]. It is determined by averaging all the samples collected at each sampling location for the past 12 months. The level of [TTHM/HAA5] averaged at one of our system’s locations for [provide date range] was [level].

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. If a situation arises where the water is no longer safe to drink, you will be notified within 24 hours.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water.

What does this mean?

This is not an emergency. If it had been an emergency, you would have been notified within 24 hours.

[TTHM are four volatile organic chemicals] [HAA5 are five haloacetic acid compounds] which form when disinfectants react with natural organic matter in the water.

[*People who drink water containing trihalomethanes in excess of the MCL over many years may experience problems with their liver, kidneys, or central nervous system, and may have an increased risk of getting cancer.*] OR

[*People who drink water containing haloacetic acids in excess of the MCL over many years may have an increased risk of getting cancer.*]

What is being done?

[Describe corrective action.] We anticipate resolving the problem within [estimated time frame] (or the problem was resolved on [give date]).

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for SWTR Failure to Filter – Template 2-4

Template on Reverse

Since surface water treatment technique violations require Tier 2 public notification, you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with surface water treatment technique violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- Our treatment plant needs upgrades to meet the requirements.

- We are installing filtration. We expect that the filtration system will be operational by [month, year].

- We are monitoring for turbidity (cloudiness), disinfectant levels, and the presence of bacteria.

- We continue to meet the standards for these measurements.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

SWTR Failure to Filter Notice – Template 2-4

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Does Not Meet Treatment Requirements

Our water system recently violated a drinking water requirement. Although this situation does not require that you take immediate action, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

On [give date], the [state] ordered us to filter the water in addition to disinfecting. We are required to install this filtration because we do not have an adequate watershed control program in place. However, we have not yet installed a filtration system.

What should I do?

- You do not need to boil your water. However, if you have specific health concerns, consult your doctor. A home filter will not necessarily solve the problem, because not all home filters protect against parasites. Call NSF International at 1(800) NSF-8010 or the Water Quality Association at 1(800) 749-0234 for information on appropriate filters.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by microbes are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours. We do not know of any cases of contamination. However, until improvements are made, there is an increased chance that disease-causing organisms could contaminate the water supply. *Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.* These symptoms, however, are not caused only by organisms in drinking water. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

Filtration is the best method for removing these organisms. [Describe corrective action.]

We anticipate resolving the problem within [estimated time frame]. Until filtration is installed, you will receive a notice similar to this every three months.

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for SWTRs Turbidity Exceedance – Template 2-5

Template on Reverse

Since surface water treatment filtration treatment technique violations require Tier 2 public notification, you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

| For Exceedances of Single Turbidity Limits | For Exceedances of Monthly Turbidity Limits |
| --- | --- |
| You must consult with your state as soon as practical but within 24 hours of learning of the violation. During the consultation, the agency may choose to elevate your turbidity exceedance to Tier 1. If consultation does not occur, the violation is automatically elevated to Tier 1 (use Template 1-6). For a Tier 2 notice, describe your violation as follows in the second paragraph of the notice: “Normal turbidity levels at our plant are [number] turbidity units. A water sample taken [date] showed levels of [number] turbidity units. This was above the standard of [standard] units. Because of these high levels of turbidity, there is an increased chance that the water may contain disease-causing organisms.” | Use the following language to describe your violation and insert into the second paragraph of the template: “Water samples for [month] showed that [percentage] percent of turbidity measurements were over [standard] turbidity units – the standard is that no more than 5 percent of samples may exceed [standard] turbidity units per month. The turbidity levels are relatively low. However, their persistence is a concern. Normal turbidity levels at our plant are [number] units.” |

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information. You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with filtration treatment technique violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We added chemicals that reduce turbidity.

- We sampled both untreated and treated water for the presence of coliform bacteria.

- We monitored chlorine levels and adjusted them as needed to compensate for the filtration problems.

- We inspected and cleaned the filters.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)]. It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

SWTRs Turbidity Exceedance Notice – Template 2-5

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Does Not Meet Treatment Requirements

Our water system recently violated a drinking water requirement. Although this was not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

We routinely monitor your water for turbidity (cloudiness). This tells us whether we are effectively filtering the water supply. [Insert appropriate description of the violation from instructions on the previous page.]

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other actions. We do not know of any contamination, and none of our testing has shown disease-causing organisms in the drinking water.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by microbes are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

*Turbidity has no health effects. However, turbidity can interfere with disinfection and provide a medium for microbial growth. Turbidity may indicate the presence of disease causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms are not caused only by organisms in drinking water. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

[Describe the corrective actions and when the system returned or expects to return to compliance.]

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for SWTR Disinfection Treatment – Template 2-6

Template on Reverse

Since surface water treatment disinfection treatment technique violations require Tier 2 public notification, you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Description of the Violation

Include one of the following descriptions of violations, and modify to fit your situation.

- Not Enough Disinfection During Treatment - In order to ensure proper disinfection, water in the treatment plant must be in contact with enough chlorine or a similar disinfectant for a minimum amount of time. On [give date], this did not occur. Although chlorine quickly kills most bacteria, it is less effective against organisms such as viruses and parasites. For this reason, water needs to mix with chlorine for a longer time period to kill such organisms. The amount of time necessary, or the “contact time,” depends on the amount of disinfectant in the water and the temperature of the water.

- Disinfectant Residual - We routinely monitor for disinfectant residual in the distribution system. This measurement tells us whether we are effectively disinfecting the water supply. Disinfectant residual is the amount of chlorine or related disinfectant present in the pipes of the distribution system. If the amount of disinfectant is too low, organisms could grow in the pipes.

- Monthly exceedance - During the months of [give dates], disinfectant residual was undetectable in more than 5% of samples. The standard is that disinfectant may be undetectable in no more than 5% of samples each month for two months in a row.

- Single exceedance - On [give date], disinfectant levels dropped below 0.2 milligrams per liter (mg/L) for [give number of hours] hours. The standard is that levels may not drop below 0.2 mg/L for more than four hours.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with disinfection treatment technique (TT) violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are sampling/we sampled both untreated and treated water for the presence of coliform bacteria.

- We are sampling/we sampled disinfectant levels and will adjust/adjusted the amount of disinfectant added as necessary to maintain adequate levels.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)]. It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

SWTR Disinfection Treatment Notice – Template 2-6

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Does/Did Not Meet Treatment Requirements

Our water system recently violated a drinking water requirement. Although this situation does not require that you take immediate action, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

[Describe the violation - use descriptions from instructions on reverse.]

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by microbes are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours. Tests taken during this same time period did not indicate the presence of bacteria in the water.

*Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

[Describe corrective action.]

[Disinfectant residual levels/contact times] so far this month have met all requirements.

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Failure to Install Corrosion Control – Template 2-7

Template on Reverse

Since lead and copper treatment technique violations require Tier 2 public notification, you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Description of the Violation

This template is written for systems which are required to install corrosion control after exceeding lead action levels. The Lead and Copper Rule requires some large systems to install corrosion control even if they have never exceeded the lead action level. You may need to modify the template if this applies to you. The following may help you explain the violation:

- This is a treatment violation, but it does not mean there is lead in your drinking water. However, it is important that we take measures to control lead levels in the water, because ingesting lead can cause serious health consequences.

If the delay in installation is related to outside circumstances, such as funding, you should explain the delay. Consumers may be more supportive of rate increases or may pressure local authorities to provide funds if they understand the circumstances.

Corrective Action

In your notice, describe corrective actions you are taking. You can use the following language, if appropriate, or develop your own text:

- We conducted a lead public education program in [month, year]. You should have received a brochure explaining in more detail steps you can take to reduce exposure until corrosion control is in place.

If consumers ask for information on testing their water, you should have on hand the names of laboratories consumers can call. Tell consumers to call NSF International at 1(800) NSF-8010 or the Water Quality Association at 1(800) 749-0234 for information on appropriate filters. For more information on lead, have consumers visit the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

Failure to Install Corrosion Control Notice – Template 2-7

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Water Contains High Levels of Lead

Our water system recently violated a drinking water requirement. Even though this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

We routinely sample water at consumers’ taps for lead. The tests show lead levels in the water above the limit, or “action level,” so we are required to install corrosion control treatment. This treatment helps prevent lead in the pipes from dissolving into the water. Corrosion control should have been installed by [give date], but installation is incomplete.

What should I do?

Listed below are some steps you can take to reduce your exposure to lead:

- Call us at the number below to find out how to get your water tested for lead.

- Find out whether your pipes contain lead or lead solder.

- Run your water for 15-30 seconds or until it becomes cold before using it for drinking or cooking. This flushes any standing lead from the pipes.

- Don’t cook with or drink water from the hot water tap; lead dissolves more easily into hot water.

- Do not boil your water to remove lead. Boiling water makes the lead more concentrated – the lead remains when the water evaporates.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours. Typically, lead enters water supplies by leaching from lead or brass pipes and plumbing components. New lead pipes and plumbing components containing lead are no longer allowed for this reason. However, many older homes may contain lead pipes. Your water is more likely to contain high lead levels if water pipes in or leading to your home are made of lead or contain lead solder.

*Infants and children who drink water containing lead in excess of the action level could experience delays in their physical or mental development. Children could show slight deficits in attention span and learning abilities. Adults who drink this water over many years could develop kidney problems or high blood pressure.*

What is being done?

[Describe corrective action.] Corrosion control will be in place by [give date].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Failure to Install Source Water Treatment – Template 2-8

Template on Reverse

Since lead and copper treatment technique violations require Tier 2 public notification, you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Description of the Violation

This template is written for systems which are required to install source water treatment after exceeding lead action levels. The following may help you explain the violation:

- This is a treatment violation, but it does not mean there is lead in your drinking water. However, it is important that we take measures to control lead levels in the water, because ingesting lead can cause serious health consequences.

If the delay in installation is related to outside circumstances, such as funding, you should explain the delay. Consumers may be more supportive of rate increases or may pressure local authorities to provide funds if they understand the circumstances.

Corrective Action

In your notice, describe corrective actions you are taking. You can use the following language, if appropriate, or develop your own text:

- We conducted a lead public education program in [month, year]. You should have received a brochure explaining in more detail steps you can take to reduce exposure until the source water treatment is in place.

If consumers ask for information on testing their water, you should have on hand the names of laboratories consumers can call. Tell consumers to call NSF International at 1(800) NSF-8010 or the Water Quality Association at 1(800) 749-0234 for information on appropriate filters. For more information on lead, have consumers visit the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

Failure to Install Source Water Treatment Notice – Template 2-8

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Water Contains High Levels of Lead

Our water system recently violated a drinking water requirement. Even though this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

We routinely sample water at consumers’ taps for lead. The tests show lead levels in the water above the limit, or “action level,” so we are required to install source water treatment. This treatment removes lead found in the source water from the water you receive. Source water treatment should have been installed by [give date], but installation is incomplete.

What should I do?

Listed below are some steps you can take to reduce your exposure to lead:

- Call us at the number below to find out how to get your water tested for lead.

- Find out whether your pipes contain lead or lead solder.

- Run your water for 15-30 seconds or until it becomes cold before using it for drinking or cooking. This flushes any standing lead from the pipes.

- Don’t cook with or drink water from the hot water tap; lead dissolves more easily into hot water.

- Do not boil your water to remove lead. Boiling water makes the lead more concentrated – the lead remains when the water evaporates.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours. Typically, lead enters water supplies by leaching from lead or brass pipes and plumbing components. New lead pipes and plumbing components containing lead are no longer allowed for this reason. However, many older homes may contain lead pipes. Your water is more likely to contain high lead levels if water pipes in or leading to your home are made of lead or contain lead solder.

*Infants and children who drink water containing lead in excess of the action level could experience delays in their physical or mental development. Children could show slight deficits in attention span and learning abilities. Adults who drink this water over many years could develop kidney problems or high blood pressure.*

What is being done?

[Describe corrective action.] Source water treatment will be in place by [give date].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Lead and Copper Rule (LCR) Failure to Meet Reporting Requirements for Public Education – Template 2-9

Template on Reverse

Since lead and copper treatment technique violations require Tier 2 public notification, you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Description of the Violation

This template is written for systems which have not met timing, content, delivery and/or State reporting requirements for Public Education. The Lead and Copper Rule requires a water system that exceeds the lead action level based on tap water samples collected in accordance with § 141.86 to deliver the public education materials to their consumers. You may need to modify the template if this applies to you. The following may help you explain the violation:

- This is a treatment violation, but it does not mean there is lead in your drinking water. However, it is important that we take measures to control lead levels in the water, because ingesting lead can cause serious health consequences.

Corrective Action

In your notice, describe corrective actions you are taking. You can use the following language, if appropriate, or develop your own text:

- We will have begun conducting a lead public education program. You should receive a brochure explaining in more detail steps you can take to reduce exposure.

If consumers ask for information on testing their water, you should have on hand the names of laboratories consumers can call. Tell consumers to call NSF International at 1(800) NSF-8010 or the Water Quality Association at 1(800) 749-0234 for information on appropriate filters. For more information on lead, have consumers visit the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

Lead and Copper Rule (LCR) Failure to Meet Reporting Requirements for Public Education – Template 2-9

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Water Contains High Levels of Lead

Our water system recently violated a drinking water requirement. Even though this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

We routinely sample water at consumers’ taps for lead. (Water System Name] found elevated levels of lead in drinking water in some homes/buildings. Lead can cause serious health problems, especially for pregnant women and young children. Please read this information closely to see what you can do to reduce lead in your drinking water.

What should I do?

Listed below are some steps you can take to reduce your exposure to lead:

- Call us at the number below to find out how to get your water tested for lead.

- Find out whether your pipes contain lead or lead solder.

- Run your water for 15-30 seconds or until it becomes cold before using it for drinking or cooking. This flushes any standing lead from the pipes.

- Don’t cook with or drink water from the hot water tap; lead dissolves more easily into hot water.

- Do not boil your water to remove lead. Boiling water makes the lead more concentrated – the lead remains when the water evaporates.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours. Typically, lead enters water supplies by leaching from lead or brass pipes and plumbing components. New lead pipes and plumbing components containing lead are no longer allowed for this reason. However, many older homes may contain lead pipes. Your water is more likely to contain high lead levels if water pipes in or leading to your home are made of lead or contain lead solder.

*Infants and children who drink water containing lead in excess of the action level could experience delays in their physical or mental development. Children could show slight deficits in attention span and learning abilities. Adults who drink this water over many years could develop kidney problems or high blood pressure.*

What is being done?

[Describe corrective action.]

For more information, please contact [name of contact] at [phone number] or [mailing address].

For more information on reducing lead exposure around your home/building and the health effects of lead, visit EPA's Web site at http://www.epa.gov/lead or contact your health care provider.

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Failure to Replace Lead Service Lines – Template 2-10

Template on Reverse

Since lead and copper treatment technique violations require Tier 2 public notification, you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Description of the Violation

This template is written for systems which are required to replace lead service lines after exceeding lead action levels. The following may help you explain the violation:

- This is a treatment violation, but it does not mean there is lead in your drinking water. However, it is important that we take measures to control lead levels in the water, because ingesting lead can cause serious health consequences.

If the delay in installation is related to outside circumstances, such as funding, you should explain the delay. Consumers may be more supportive of rate increases or may pressure local authorities to provide funds if they understand the circumstances.

Corrective Action

In your notice, describe corrective actions you are taking. You can use the following language, if appropriate, or develop your own text:

- We conducted a lead public education program in [month, year]. You should have received a brochure explaining in more detail steps you can take to reduce exposure until lead service lines are replaced and the system is in compliance with the Lead and Copper Rule.

If consumers ask for information on testing their water, you should have on hand the names of laboratories consumers can call. Tell consumers to call NSF International at 1(800) NSF-8010 or the Water Quality Association at 1(800) 749-0234 for information on appropriate filters. For more information on lead, have consumers visit the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

Failure to Replace Lead Service Lines Notice – Template 2-10

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Water Contains High Levels of Lead

Our water system recently violated a drinking water requirement. Even though this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

We routinely sample water at consumers’ taps for lead. The tests show lead levels in the water above the limit, or “action level,” so we are required to replace lead service lines in the distribution system. Lead service line replacement should have been installed by [give date], but installation is incomplete.

What should I do?

Listed below are some steps you can take to reduce your exposure to lead:

- Call us at the number below to find out how to get your water tested for lead.

- Find out whether your pipes contain lead or lead solder.

- Run your water for 15-30 seconds or until it becomes cold before using it for drinking or cooking. This flushes any standing lead from the pipes.

- Don’t cook with or drink water from the hot water tap; lead dissolves more easily into hot water.

- Do not boil your water to remove lead. Boiling water makes the lead more concentrated – the lead remains when the water evaporates.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours. Typically, lead enters water supplies by leaching from lead or brass pipes and plumbing components. New lead pipes and plumbing components containing lead are no longer allowed for this reason. However, many older homes may contain lead pipes. Your water is more likely to contain high lead levels if water pipes in or leading to your home are made of lead or contain lead solder.

*Infants and children who drink water containing lead in excess of the action level could experience delays in their physical or mental development. Children could show slight deficits in attention span and learning abilities. Adults who drink this water over many years could develop kidney problems or high blood pressure.*

What is being done?

[Describe corrective action.] Replacement of the lead service will be replaced [give date].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for LT2ESWTR Uncovered Finished Water Reservoir Treatment – Template 2-11

Template on Reverse

A system’s failure to cover an uncovered finished water reservoir, provide treatment of the reservoir’s discharge, or be in compliance with a state-approved schedule to cover the reservoir(s) or treat the reservoir(s) discharge by April 1, 2009, is a treatment technique violation and requires Tier 2 public notification. You must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.  You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with this type of LT2ESWTR treatment technique violation. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are currently working with state officials to revise our schedule for covering our uncovered finished water storage reservoir. We expect to have the state-approved schedule in place by [give date] and the reservoir covered by [give date]. Until our schedule receives state approval, you will receive a notice similar to this every three months.

- We are currently working with state officials to revise our schedule for treating the discharge from our uncovered finished water storage reservoir. We expect to have the state-approved schedule in place by [give date] and treatment in place by [give date]. Until the schedule receives state approval, you will receive a notice similar to this every three months.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)]. It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

LT2ESWTR Uncovered Finished Water Reservoir Treatment Notice –

Template 2-11

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

Uncovered Finished Water Reservoir Violation at [Water System Name]

Our water system recently violated a drinking water requirement. Although this situation is not an emergency, as our customers, you have a right to know what happened and what we did (are doing) to correct this situation.

Water systems with uncovered finished water reservoirs are required to cover these reservoirs, treat the water from these reservoirs, or be in compliance with a state-approved schedule to cover the reservoir(s) or provide treatment. We have failed to meet this requirement.

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by microbes are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

Uncovered finished water reservoirs are a pathway for contaminants to enter the drinking water supply. Therefore, it is necessary to cover the reservoirs or treat the discharge. *Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

[Describe corrective action.] We anticipate resolving the problem within [estimated time frame].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for LT2ESWTR Filtered System Failure to Provide or Install an Additional Level of Treatment – Template 2-12

Template on Reverse

A filtered system’s failure to provide or install an additional level of treatment appropriate for its LT2ESWTR bin classification by the required date, is a treatment technique (TT) violation and requires Tier 2 public notification. You must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with LT2ESWTR treatment technique violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are currently working to [install treatment/adjust our treatment processes] to achieve the required level of additional treatment.

- We are currently working with state officials and our engineers to finalize plans for water treatment that will satisfy this requirement once it is installed.

- We have [installed treatment/made adjustments to our treatment processes] and are now achieving the required level of additional treatment.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

LT2ESWTR Filtered System Failure to Provide or Install an Additional Level of Treatment Notice – Template 2-12

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Provide the Appropriate Level of Treatment Within Required Time Frame

Our water system recently violated a drinking water requirement. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

Cryptosporidium is a disease-causing microorganism that has been found in our raw water source. Our water system [give system name] was required to provide an additional level of treatment to address Cryptosporidium by [provide date]. We failed to do this by the required date.

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

*Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

[Describe corrective action.] We expect to achieve this additional level of treatment by [give date].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for LT2ESWTR Unfiltered System Failure to Achieve Required Inactivation Level – Template 2-13

Template on Reverse

Based on Cryptosporidium monitoring results, an unfiltered system must meet inactivation requirements for Cryptosporidium by a required date. Failure to do this is a treatment technique violation and requires Tier 2 public notification. You must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with LT2ESWTR treatment technique violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are currently working to adjust our treatment processes to achieve the required level of treatment.

- We are currently working with state officials and our engineers to finalize plans for water treatment that will satisfy this requirement once it is installed.

- We have made adjustments to our treatment processes and are now achieving the required level of treatment.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

LT2ESWTR Unfiltered System Failure to Achieve Required Inactivation Level Notice – Template 2-13

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Provide the Appropriate Level of Treatment Within Required Time Frame

Our water system recently violated a drinking water requirement. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

Cryptosporidium is a disease-causing microorganism that may be in our raw water source. Our water system [give system name] was required to provide a specified level of treatment to address Cryptosporidium by [provide date]. We failed to do this by the required date.

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

*Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

[Describe corrective action.] We expect to achieve this specified level of treatment by [give date].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for LT2ESWTR Filtered System Failure to Achieve Required Treatment Credit – Template 2-14

Template on Reverse

Based on Cryptosporidium monitoring results, some filtered systems are required to achieve additional treatment credit using a microbial toolbox option. A filtered system’s failure to achieve required treatment credit to meet the bin classification requirements, is a treatment technique violation and requires Tier 2 public notification. You must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with LT2ESWTR treatment technique violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are taking steps to ensure the required level of treatment can be maintained at all times.

- The reason for the lapse in treatment has been identified and addressed.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-11 for a “problem corrected” notice template.

LT2ESWTR Filtered System Failure to Achieve Required Treatment Credit Notice – Template 2-14

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Maintain the Required Level of Treatment

Our water system recently violated a drinking water requirement. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

Cryptosporidium is a disease-causing microorganism that has been found in our raw water source. Our water system [give system name] was required to maintain a specified level of additional treatment to address Cryptosporidium. During [provide date range], our system fell below this level.

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

*Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

[Describe corrective action and when the system returned or expects to return to compliance.]

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for LT2ESWTR Unfiltered System Failure to Maintain Required Inactivation Level – Template 2-15

Template on Reverse

Based on Cryptosporidium monitoring results, an unfiltered system must maintain specified inactivation levels to ensure adequate treatment for Cryptosporidium. Failure to do this is a treatment technique violation and requires Tier 2 public notification. You must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with LT2ESWTR treatment technique violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are taking steps to ensure the required level of treatment can be maintained at all times.

- The reason for the lapse in treatment has been identified and addressed.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

LT2ESWTR Unfiltered System Failure to Maintain Required Inactivation Level Notice – Template 2-15

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Maintain the Required Level of Treatment

Our water system recently violated a drinking water requirement. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

Cryptosporidium is a disease-causing microorganism that may be in our raw water source. Our water system [give system name] was required to maintain a specified level of treatment to address Cryptosporidium. During [provide date range], our system fell below this level.

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

*Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

[Describe corrective action and when the system returned or expects to return to compliance.]

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for LT2ESWTR Unfiltered System Failure to Install Second Disinfectant – Template 2-16

Template on Reverse

An unfiltered system’s failure to install a second disinfectant to treat for Cryptosporidium by the required date, is a treatment technique violation and requires Tier 2 public notification. You must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by unfiltered water systems with LT2ESWTR treatment technique violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We will install a second disinfectant by [provide date] to satisfy this requirement.

- We are currently working with state officials and our engineers to finalize plans for water treatment that will satisfy this requirement once it is installed.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

LT2ESWTR Unfiltered System Failure to Install Second Disinfectant Notice – Template 2-16

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Provide the Appropriate Level of Treatment Within Required Time Frame

Our water system recently violated a drinking water requirement. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

Our unfiltered water system [give system name] was required to use a minimum of two disinfectants to meet our treatment requirement by [give date]. We failed to have the new [ultraviolet light (U.V.)/ chlorine dioxide/ozone] disinfection system installed by the required date.

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by microbes are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

*Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

We expect to have the [U.V./chlorine dioxide/ozone] disinfection system installed by [give date]. We continue to disinfect the water with chlorine as we have in the past.

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Filter Backwash Recycling Rule Treatment Technique Violation – Template 2-17

Template on Reverse

Since failure to recycle spent filter backwash, thickener supernatant, and liquids from dewatering processes before all processes of the direct filtration treatment train; failure to obtain state approval for the use of an alternative recycle location; or failure to pursue capital improvements are violations that require Tier 2 public notification, you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Do not use overly technical terminology when describing treatment methods. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- On [give date], we will no longer recycle our filter backwash and will not be in violation of this requirement.

- On [give date], the filter backwash will be piped to the beginning of our water treatment process and we will no longer be in violation of this requirement.

- On [give date], the filter backwash and all other current recycle flows will be discharged to waste as approved by the state.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

Filter Backwash Recycling Rule Treatment Technique Violation 
Notice – Template 2-17

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Meet Treatment Requirements

Our water system recently violated a drinking water requirement. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

The Filter Backwash Recycling Rule requires water systems that recycle liquids used for water treatment to recycle these liquids through the system’s existing filtration system or to an alternate location approved by the state. Our system did not return its water treatment recycle flows to the required location or a state-approved location by the required date of [give date].

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by microbes are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

*Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

[Describe corrective action.] We anticipate resolving the problem within [estimated time frame].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Failure of a Seasonal System to Complete Start-Up Procedures – Template 2-18

Template Below

Description of Violation or Situation

A seasonal NCWS that fails to follow state-approved start-up procedures prior to serving water to the public after the system had been out of service has incurred a treatment technique (TT) violation. This violation requires Tier 2 public notification. Start-up procedures are approved by the state and may include, but are not limited to, activities such as:

- Inspecting all water system components, including source(s), treatment components, distribution lines, and storage tanks and addressing any issues.

- Activating the source and thoroughly flushing water through all pipes in the water system.

- Draining and re-filling storage tanks.

- Chlorinating the water in the system and leaving the chlorinated water in the system for at least 24 hours before flushing the water to waste.

- Collecting water samples at key locations within the system and ensuring the water system is not contaminated by bacteria.

You must provide public notice to persons served as soon as practical but no later than 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

NCWSs must use one of the following methods to deliver the notice to consumers [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, EPA recommends printing your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below). All posted notices must remain in place for as long as the violation or situation persists but in no case for less than seven days, even if the violation or situation is resolved.

Mandatory Language

Mandatory language on health effects (from 40 CFR 141.205(d)(2)) must be included as written if the required start-up procedure includes collecting coliform bacteria samples. The mandatory language is provided below and is presented in this notice in italics with an asterisk on each end. You will need to update the information presented in brackets with the appropriate information. If monitoring is not required as part of the start-up procedures the italicized mandatory language should be omitted and language must be used to appropriately describe the violation.

*We are required to monitor your drinking water for specific contaminants on a regular basis. Results of regular monitoring are an indicator of whether or not your drinking water meets health standards. During [compliance period], we “did not complete all monitoring or testing” for [contaminant(s)], and therefore cannot be sure of the quality of your drinking water during that time.*

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on each end.

Corrective Action

In your notice, you must describe corrective actions you are taking [40 CFR 141.205(a)(7)] and when you expect to return to compliance or resolve the situation [40 CFR 141.205(a)(8)]. Do not use overly technical terminology when describing treatment methods. Listed below are some steps commonly taken by water systems with TT violations relating to incomplete start-up procedures. Depending on the corrective action you are taking, you can use the following statement, if appropriate, or develop your own text:

- We have completed the required start-up procedures and provided the appropriate certification to the state. The procedures included [describe what you did, for example, flushed the system, disinfected the system, collected total coliform bacteria samples, etc.].

- We have collected [number] coliform bacteria samples as required and [number] sample results were negative for total coliform bacteria.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

Revised Total Coliform Rule (RTCR) Failure of a Seasonal System to Complete Start-Up Procedures – Template 2-18

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Complete Seasonal Start-Up Procedures

Before we open each year and serve water to the public we are required to complete certain start-up procedures to make sure the water we provide is safe to drink. This year we failed to [describe what you failed to do, for example, disinfect the water system or collect the required number of coliform bacteria samples] before providing water. As our customers, you have a right to know what happened and what we are doing to correct this situation.

Inadequately treated or inadequately protected water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches.

*We are required to monitor your drinking water for specific contaminants on a regular basis. Results of regular monitoring are an indicator of whether or not your drinking water meets health standards. During [compliance period], we “did not complete all monitoring or testing” for [contaminant(s)], and therefore cannot be sure of the quality of your drinking water during that time.*

What should I do?

- If you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare provider about drinking this water. You should also seek advice from your healthcare provider about using the water if you have an infant. General guidelines on ways to lessen the risk of infection by bacteria and other disease-causing organisms are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency, as we have completed the start-up procedures. If it had been, you would have been notified within 24 hours.

Failure to perform the required start-up procedures prior to serving water to the public has the potential to distribute contaminated water. When our system shuts down operation, the lack of pressure in our pipes can allow the entry of bacteria and other disease-causing microorganisms into the drinking water. By performing start-up procedures such as flushing the pipes, disinfecting the water, and collecting a coliform bacteria sample before we open, we can be sure that we are providing you with safe water.

What is being done?

[Describe corrective action including when you returned to compliance or resolved the situation].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

This page intentionally left blank.

Instructions for Monitoring Violation Elevated to Tier 2 – Template 2-19

Template on Reverse

If your state has designated a failure to monitor as a violation requiring Tier 2 public notification [40 CFR 141.203(a)], you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Monitoring violations are Tier 3 by default, but may be elevated to Tier 2 by your state. You may elevate the public notice to Tier 2 yourself as well. You must issue a repeat notice every three months for as long as the violation persists. Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a)and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you took or are taking. Listed below are some steps commonly taken by water systems with monitoring violations. You can use the following language, if appropriate, or develop your own text:

- We have since taken the required samples, as described in the last column of the table above. The samples showed we are meeting drinking water standards.

- We have since taken the required samples, as described in the last column of the table above. The sample for [contaminant] exceeded the limit. [Describe corrective action; use information from public notice prepared for violating the limit.]

- We plan to take the required samples [expected time frame], as described in the last column of the table above.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

Monitoring Violation Elevated to Tier 2 Notice – Template 2-19

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

Monitoring Requirements Not Met for [Water System Name]

Our water system recently violated a drinking water requirement. Although this incident was not an emergency, as our customers, you have a right to know what happened and what we did (are doing) to correct this situation.

*We are required to monitor your drinking water for specific contaminants on a regular basis. Results of regular monitoring are an indicator of whether or not our drinking water meets health standards. During [compliance period] we [‘did not monitor or test’ or ‘did not complete all monitoring or testing’] for [contaminant(s)] and, therefore, cannot be sure of the quality of your drinking water during that time.*

What should I do?

There is nothing you need to do at this time.

The table below lists the contaminant(s) we did not properly test for, how often we are supposed to sample for [this contaminant/these contaminants] and how many samples we are supposed to take, how many samples we took, when samples should have been taken, and the date on which follow-up samples were (or will be) taken.

| Contaminant | Required sampling frequency | Number of samples taken | When samples should have been taken | When samples were taken |
| --- | --- | --- | --- | --- |
| Nitrate (example) | One sample annually | 0 | 2008 | February 2009 |

What is being done?

[Describe corrective action.]

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Failure to Comply with the Conditions of an Exemption – Template 2-20

Template on Reverse

Since failure to comply with the conditions of an exemption requires Tier 2 public notification, you must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. You must issue a repeat notice every three months for as long as the violation persists. Your state may have more stringent or alternative requirements for exemption violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below is an example of a step that might be taken in response to this violation. You can include the following statement, if appropriate, or develop your own text:

- We have agreed upon a new deadline of [give date] to secure the funding needed in order to purchase [contaminant] removal treatment equipment. Once we have secured funding, we will purchase and begin installation of the selected treatment technology by [estimated time frame].

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

Failure to Comply with the Conditions of an Exemption Notice – Template 2-20

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Meet the Conditions of a [contaminant] Exemption

Our water system recently violated one of the conditions of our [contaminant] exemption. An exemption allows eligible systems additional time to build capacity in order to comply with a drinking water standard. [Water System Name] was granted an exemption for the [contaminant] standard so that we could raise funds for a new treatment process to remove [contaminant]. As a condition of the exemption, we agreed to secure funding by [give date]. We did not meet this deadline.

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. You may continue to drink the water. If a situation arises where the water is no longer safe to drink, you will be notified within 24 hours. We will post this information on our Web site at [system Web site].

What does this mean?

This is not an emergency. If it had been an emergency, you would have been notified within 24 hours.

(Example language provided for arsenic. Enter required language from 40 CFR Appendix B to 40 CFR 141 Subpart Q for your specific contaminant.)

[*Some people who drink water containing arsenic in excess of the MCL over many years may have an increased risk of getting cancer.*]

What is being done?

[Describe corrective action.] We anticipate resolving the problem within [estimated time frame] OR The problem was resolved on [give date].

For more information, please contact [name of contact] at [phone number] or [mailing address]. You can also visit our Web site at www.ourwatersystem.com.

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for GWR Failure to Maintain 4-log Treatment of Viruses – Template 2-21

Template on Reverse

A ground water system’s failure to maintain required 4-log treatment of viruses for a period greater than four hours is a treatment technique violation that requires Tier 2 public notification. You must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with Ground Water Rule treatment technique violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We are increasing disinfection to maintain our proper residual levels.

- We are pursuing improvements to our treatment system so that we can maintain required treatment at all times.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

GWR Failure to Maintain 4-log Treatment of Viruses Notice – Template 2-21

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failure to Meet Treatment Requirements

Our water system recently violated a drinking water requirement. Although this incident was not an emergency, as our customers, you have a right to know what happened and what we did (are doing) to correct this situation.

We are required to [treat/disinfect] our drinking water source [name source]. From [give date] to [give date] we did not meet one or more of our treatment requirements [provide sufficient levels of disinfectant/provide adequate contact time/meet alternative treatment criteria] due to [describe issue, e.g., malfunctioning equipment].

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by microbes are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

*Inadequately treated or inadequately protected water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

While we have not detected any evidence of contamination in, or other health threats to, our source water, we are still committed to restoring the required level of treatment to the water from [source] to eliminate the threat of contamination.

What is being done?

[Describe corrective action.] We anticipate resolving the problem within [estimated time frame]. OR The problem was resolved on [give date].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for GWR Failure to Take Corrective Action Within Required Time Frame – Template 2-22

Template on Reverse

A system’s failure to take corrective action within the required timeframe or be in compliance with a state-approved corrective action plan and schedule for a fecal indicator-positive ground water source sample or significant deficiency under GWR is a treatment technique violation and requires Tier 2 public notification. You must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. Your state may have more stringent requirements for treatment technique violations (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with Ground Water Rule treatment technique violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- Although we did not meet our deadline, we are now in consultation with the state to develop a corrective action plan.

- The [source of contamination/significant deficiency] has been identified and addressed.

- We have implemented a short-term plan to address the immediate issue while we pursue the long-term solution.

Repeat Notices

For repeat notices, you should state how long the violation has been ongoing and remind consumers of when you sent out any previous notices. If you are making progress, describe it. Alternatively, if funding or other issues are delaying progress, let consumers know.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

GWR Failure to Take Corrective Action Within Required Time Frame 
Notice – Template 2-22

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to [Correct a Significant Deficiency/Address a Fecal Indicator-Positive Source Sample] Within Required Time Frame.

Our water system recently violated a drinking water requirement. Although this incident was not an emergency, as our customers, you have a right to know what happened and what we did (are doing) to correct this situation.

[A sanitary survey conducted on [give date] by the [insert state] found [describe significant deficiency in our water system]] OR

[Sampling conducted at our ground water source on [given date(s)] found indication of fecal contamination of our source(s)].

As required by Environmental Protection Agency’s (EPA’s) Ground Water Rule (GWR), we were required to take action to [correct this deficiency/address the fecal-indicator positive source sample]. However, we failed to take this action by the deadline established by [insert state name].

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions, unless further direction is received. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by microbes are available from on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

*Inadequately treated or inadequately protected water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

[Describe corrective action.] We anticipate resolving the problem within [estimated time frame] (or the problem was resolved on [give date]).

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Failure to Perform any Level 1 Assessment or a Level 2 Assessment that is Not Triggered by E. coli MCL Violations or the Related Corrective Actions – Template 2-23

Template Below

Description of Violation or Situation

A PWS triggers a Level 1 assessment when:

- For systems taking 40 or more samples (including routine and repeat samples) per month, the public water system exceeds 5.0 percent total coliform-positive samples for the month;

- For systems taking fewer than 40 samples (including routine and repeat samples) per month, the public water system has two or more total coliform-positive samples in the same month; or

- The PWS fails to take every required repeat sample after any single routine total coliform-positive sample.

A PWS triggers a Level 2 assessment when:

- The PWS has a second Level 1 assessment TT trigger within a rolling 12-month period unless the state has determined a likely reason for the total coliform-positive samples that caused the initial Level 1 assessment TT trigger, and the state establishes that the system has fully corrected the problem; or,

- For PWSs with approved reduced annual monitoring, the system has a Level 1 assessment TT trigger in two consecutive years.

TT violations related to triggered assessments occur when any public water system has:

- Failed to conduct the triggered Level 1 or Level 2 assessment within 30 days after learning that it has exceeded the trigger; or

- Failed to correct any sanitary defect found through a Level 1 or Level 2 assessment within 30 days or in accordance with a schedule acceptable to the state.

TT violations related to any Level 1 assessment or to a Level 2 assessment that is not triggered by an E. coli MCL violation require similar Tier 2 public notice. TT violations for Level 2 assessments that are triggered by E. coli MCL violations require different mandatory health effects language and are addressed in Template 2-24.

You must provide public notice to persons served as soon as practical but no later than 30 days after you learn of the violation [40 CFR 141.203(b)]. You must issue a repeat notice every three months for as long as the violation persists. Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods to deliver the notice to consumers [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods to deliver the notice to consumers [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, EPA recommends printing your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below). All posted notices must remain in place for as long as the violation or situation persists but in no case for less than seven days, even if the violation or situation is resolved.

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on each end.

*Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other, potentially harmful, waterborne pathogens may be present or that a potential pathway exists through which contamination may enter the drinking water distribution system. We found coliforms indicating the need to look for potential problems in water treatment or distribution. When this occurs, we are required to conduct assessments to identify problems and to correct any problems that are found.*

You are also required to include one or both of the following statements, also presented in this notice in italics with an asterisk on each end, as appropriate for the violation:

*We failed to conduct the required assessment.*

*We failed to correct all identified sanitary defects that were found during the assessment that we conducted.*

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also provided below and presented in this notice in italics with an asterisk on each end.

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

Corrective Action

The specific cause(s) of total coliforms in the distribution system that triggered the assessment will likely differ from system to system. Your notice must describe the corrective action(s) you are taking [40 CFR 141.205(a)(7)] to address the TT violation and when you expect to return to compliance or resolve the situation [40 CFR 141.205(a)(8)]. You can use one or more of the following statements, if appropriate, or develop your own text.

- We have begun to correct the sanitary defect(s) identified during an assessment of our water system by taking the following corrective actions: [Describe corrective actions].

- To ensure that our water system is protected against contamination, we are working with the state to implement the following corrective actions: [Describe corrective actions].

- We completed the required assessment and identified the cause of the sanitary defect to be addressed [describe the issue or problem found, for example, damage to the storage tank, a missing vent screen, etc.] We are currently correcting the problem on a schedule approved by [State Department of Public Health].

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved. See Template 1-6 of the Revised Public Notification Handbook (2nd Revision of Document: EPA 816-R-09-013, March 2010) and Template NC-7 of the Public Notification Handbook for Transient Non-community Water Systems, EPA 816-R-09-009, March 2010 for a “problem corrected” notice template.

Revised Total Coliform Rule (RTCR) Failure to Perform any Level 1 Assessment or a Level 2 Assessment that is Not Triggered by E. coli MCL Violations or the Related Corrective Actions – Template 2-23

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Perform Activities Required to Address Coliform Bacteria Contamination of the Water System

During recent routine monitoring, our water system tested positive for total coliforms. *Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other, potentially harmful, waterborne pathogens may be present or that a potential pathway exists through which contamination may enter the drinking water distribution system. We found coliforms indicating the need to look for potential problems in water treatment or distribution.

When this occurs, we are required to conduct assessments to identify problems and to correct any problems that are found.* [Describe the TT violation, using the mandatory language of *We failed to conduct the required assessment* by [Enter date the assessment was due] and/or *We failed to correct all identified sanitary defects that were found during the assessment(s)* by [Enter date correction was due].]

As our customers, you have a right to know what happened and what we are doing to correct this situation.

What should I do?

- You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare provider about drinking this water. You should also seek advice from your healthcare provider about using the water if you have an infant. General guidelines on ways to lessen the risk of infection by bacteria and other disease-causing organisms are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

Since total coliform bacteria are generally not harmful themselves, this is not an emergency. If it had been you would have been notified within 24 hours.

Failure to identify and correct the defects has the potential to cause continued distribution system contamination. Inadequately treated or inadequately protected water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches.

What is being done?

[Describe corrective action including when your water system expects to return to compliance or resolve the violation].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Instructions for Failure to Perform a Level 2 Assessment Triggered by an E. coli MCL Violation or the Related Corrective Actions – Template 2-24

Template Below

Description of Violation or Situation

A PWS triggers a Level 2 assessment when:

- The PWS has an E. coli maximum contaminant level (MCL) violation;

A TT violation related to a Level 2 assessment that was triggered by an E. coli MCL violation requires Tier 2 public notice with mandatory health effects language stating E. coli was detected in the water system. Violations of Level 2 assessments related to E. coli require Tier 2 public notice and occur when any public water system has:

- Failed to conduct the triggered Level 2 assessment within 30 days after learning that it has exceeded the trigger; or

- Failed to correct any sanitary defect found through a Level 2 assessment within 30 days or in accordance with a schedule acceptable to the state.

TT violations for Level 2 assessments that are triggered by more than one Level 1 assessment require different mandatory health effects language and are addressed in Template 2-23.

You must provide public notice to persons served as soon as practical but no later than 30 days after you learn of the violation [40 CFR 141.203(b)]. You must issue a repeat notice every three months for as long as the violation persists. Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods to deliver the notice to consumers [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods to deliver the notice to consumers [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, EPA recommends printing your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below). All posted notices must remain in place for as long as the violation or situation persists but in no case for less than seven days, even if the violation or situation is resolved.

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on each end.

*E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Human pathogens in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a greater health risk for infants, young children, the elderly, and people with severely compromised immune systems. We violated the standard for E. coli, indicating the need to look for potential problems in water treatment or distribution. When this occurs, we are required to conduct a detailed assessment to identify problems and to correct any problems that are found.*

You are also required to include one or both of the following statements, also presented in this notice in italics with an asterisk on each end, as appropriate for the violation:

*We failed to conduct the required assessment.*

*We failed to correct all identified sanitary defects that were found during the assessment that we conducted.*

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also provided below and presented in this notice in italics with an asterisk on each end.

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

Corrective Action

The specific cause(s) of E. coli in the distribution system will likely differ from system to system. Your notice must describe the corrective action(s) you are taking [40 CFR 141.205(a)(7)] and when you expect to return to compliance or resolve the situation [40 CFR 141.205(a)(8)]. You can use one or more of the following statements, if appropriate, or develop your own text.

- We have begun to correct the sanitary defect(s) identified during an assessment of our water system by taking the following corrective actions. [Describe corrective actions].

- To ensure that our water supplies are protected against contamination, we are working with the state to implement the following corrective actions. [Describe corrective actions].

- We completed the required assessment and identified the cause of the sanitary defect to be addressed [describe the issue or problem found, for example, damage to the storage tank, a missing vent screen, etc.] We are currently correcting the problem on a schedule approved by [State Department of Public Health].

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved. See Template 1-6 of the Revised Public Notification Handbook (2nd Revision of Document: EPA 816-R-09-013, March 2010) and Template NC-7 of the Public Notification Handbook for Transient Non-community Water Systems, EPA 816-R-09-009, March 2010 for a “problem corrected” notice template.

Revised Total Coliform Rule (RTCR) Failure to Perform a Level 2 Assessment Triggered by an E. coli MCL Violation or the Related Corrective Actions – Template 2-24

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Correct a Problem Discovered During an Assessment of the Water System after Testing Positive for E. coli

During recent monitoring, our water system tested positive for E. coli. *E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Human pathogens in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a greater health risk for infants, young children, the elderly, and people with severely compromised immune systems. We violated the standard for E. coli, indicating the need to look for potential problems in water treatment or distribution.

When this occurs, we are required to conduct a detailed assessment to identify problems and to correct any problems that are found.* [Describe the TT violation, using the mandatory language of *We failed to conduct the required assessment* by [Enter date the assessment was due] and/or *We failed to correct all identified sanitary defects that were found during the assessment(s)* by [Enter date correction was due]]

As our customers, you have a right to know what happened and what we are doing to correct this situation. You were notified of the E. coli in our water on [enter date when Tier 1 public notice of the MCL violation that triggered the assessment was provided] and on [enter date] that no additional contamination has been identified and that you do not need to boil your water or take other corrective actions. Although our recent sampling has indicated coliform bacteria are absent, we are still required to assess the system and correct any defects found.

What should I do?

- You still do not need to boil your water or take corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare provider about drinking this water. You should also seek advice from your healthcare provider about using the water if you have an infant. General guidelines on ways to lessen the risk of infection by bacteria and other disease-causing organisms are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been you would have been notified within 24 hours.

Failure to identify and correct system defects has the potential to cause distribution system contamination. Inadequately treated or inadequately protected water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches.

What is being done?

[Describe corrective action including when you expect to return to compliance or resolve the situation].

For more information, please contact [name of contact] at [phone number] or [mailing address].

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Special Notice: Instructions for LT2ESWTR Failure to Conduct Required Cryptosporidium Monitoring For Any Three Months – Template 2-25

Template on Reverse

A system’s failure to conduct required Cryptosporidium monitoring of its source water for any three months is a monitoring violation that requires Tier 2 public notification and special notice. (Note: failure to collect one or two of the scheduled samples requires Tier 3 notification. See Chapter 7 for an example template.) You must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. You must issue a repeat notice every three months for as long as the violation persists. Your state may have more stringent requirements for this monitoring violation (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with LT2ESWTR monitoring violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We will begin collecting the required source water monitoring samples on [give date].

- We have since taken the required samples for source water monitoring.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

Special Notice: LT2ESWTR Failure to Conduct Required Cryptosporidium Monitoring For Any Three Months Notice – Template 2-25

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

Monitoring Requirements Not Met for [Water System Name]

Our water system recently violated a drinking water requirement. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

*We are required to monitor the source of your drinking water for Cryptosporidium. Results of the monitoring are to be used to determine whether water treatment at the [treatment plant name] is sufficient to adequately remove Cryptosporidium from your drinking water. We are required to complete this monitoring and make this determination by [required bin determination date]. We [‘‘did not monitor or test’’ or ‘‘did not complete all monitoring or testing’’] on schedule and, therefore, we may not be able to determine by the required date what treatment modifications, if any, must be made to ensure adequate Cryptosporidium removal. Missing this deadline may, in turn, jeopardize our ability to have the required treatment modifications, if any, completed by the deadline required, [give date].*

Cryptosporidium is a disease-causing microorganism that may be present in our raw water source.

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

What is being done?

[Describe corrective action.]  We will collect all required samples by [give date].

*For more information, please call [name of water system contact] of [water system name] at [phone number].*

Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.

This notice is being sent to you by [water system name]. State Water System ID#: ___________.

Date distributed: ______.

Special Notice: Instructions for LT2ESWTR Filtered System Failure to Determine and Report Bin Classification – Template 2-26

Template on Reverse

A filtered system’s failure to determine its bin classification by the required date requires a Tier 2 public notification and special notice, and a system’s failure to report its bin classification by the required date is a treatment technique violation. The template on the reverse meets both of these requirements. You must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. You must issue a repeat notice every three months for as long as the violation persists. Your state may have more stringent requirements for this monitoring violation (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with LT2ESWTR treatment technique violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We determined our bin classification on [give date] and have notified the state.

- We are in the process of determining our bin classification and will notify the state upon completion.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

Special Notice: LT2ESWTR Filtered System Failure to Determine and Report Bin Classification Notice – Template 2-26

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Determine Required Treatment Based on Cryptosporidium Level

Our water system recently violated a drinking water requirement. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

*We are required to monitor the source of your drinking water for Cryptosporidium in order to determine by [give date] whether water treatment at the [provide treatment plant name] is sufficient to adequately remove Cryptosporidium from your drinking water. We have not made this determination by the required date. Our failure to do this may jeopardize our ability to have the required treatment modifications, if any, completed by the required deadline of [give date].*

Cryptosporidium is a disease-causing microorganism that may be present in our raw water source.

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

*Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

[Describe corrective action.] We anticipate resolving the problem within [estimated timeframe] (or the problem was resolved on [give date]).

*For more information, please call [name of water system contact] of [water system name] at [phone number].*

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID#: ___________. 
Date distributed: ______.

Special Notice: Instructions on LT2ESWTR Unfiltered System Failure to Calculate and Report Mean Cryptosporidium Level – Template 2-27

Template on Reverse

An unfiltered system’s failure to calculate and report its mean Cryptosporidium level by the required date is a treatment technique (TT) violation that requires Tier 2 public notification and special notice. The template on the reverse meets both of these requirements. You must provide public notice to persons served as soon as practical but within 30 days after you learn of the violation [40 CFR 141.203(b)]. You must issue a repeat notice every three months for as long as the violation persists. Your state may have more stringent requirements for this monitoring violation (e.g., it may require you to provide water from an alternate source). Check with your state to make sure you meet all its requirements.

CWSs must use one of the following methods [40 CFR 141.203(c)]:

- Hand or direct delivery

- Mail, as a separate notice or included with the bill (if delivered within 30 days of the violation)

- Another method approved in writing by the state

NCWSs must use one of the following methods [40 CFR 141.203(c)]:

- Posting in conspicuous locations

- Hand delivery

- Mail

- Another method approved in writing by the state

In addition, both CWSs and NCWSs must use another method reasonably calculated to reach others if they would not be reached by the first method [40 CFR 141.203(c)]. Such methods could include newspapers, e-mail, or delivery to community organizations. If you mail, post, or hand deliver, print your notice on your system’s letterhead, if available.

The notice on the reverse is appropriate for mailing, posting, or hand delivery. If you modify this notice, you must still include all required public notice elements from 40 CFR 141.205(a) and leave the mandatory language unchanged (see below).

Mandatory Language

Mandatory language on health effects (from Appendix B to 40 CFR 141 Subpart Q) must be included as written and is presented in this notice in italics with an asterisk on either end. You will need to update the information presented in brackets with the appropriate information.

You must also include standard language to encourage the distribution of the public notice to all persons served, where applicable [40 CFR 141.205(d)]. This language is also presented in this notice in italics with an asterisk on either end.

Corrective Action

In your notice, describe corrective actions you are taking. Listed below are some steps commonly taken by water systems with LT2ESWTR treatment technique violations. Depending on the corrective action you are taking, you can use one or more of the following statements, if appropriate, or develop your own text:

- We determined our mean Cryptosporidium level on [give date] and have notified the state.

- We are in the process of determining our mean Cryptosporidium level and will notify the state upon completion.

After Issuing the Notice

Make sure to send a copy of each type of notice and a certification that you have met all the public notification requirements to your state within 10 days after the original or any repeat notice(s) [40 CFR 141.31(d)].

It is a good idea to inform your consumers when the violation has been resolved, especially if you have regular customers. See Template 1-10 for a “problem corrected” notice template.

Special Notice: LT2ESWTR Unfiltered System Failure to Calculate and Report Mean Cryptosporidium Level Notice – Template 2-27

IMPORTANT INFORMATION ABOUT YOUR DRINKING WATER

[Water System Name] Failed to Determine Cryptosporidium Level

Our water system recently violated a drinking water requirement. Although this is not an emergency, as our customers, you have a right to know what happened, what you should do, and what we did (are doing) to correct this situation.

*We are required to monitor the source of your drinking water for Cryptosporidium in order to determine by [give date] whether water treatment at the [provide treatment plant name] is sufficient to adequately remove Cryptosporidium from your drinking water. We have not made this determination by the required date. Our failure to do this may jeopardize our ability to have the required treatment modifications, if any, completed by the required deadline of [give date].*

Cryptosporidium is a disease-causing microorganism that may be present in our raw water source.

What should I do?

- There is nothing you need to do. You do not need to boil your water or take other corrective actions. However, if you have specific health concerns, consult your doctor.

- If you have a severely compromised immune system, have an infant, are pregnant, or are elderly, you may be at increased risk and should seek advice from your healthcare providers about drinking this water. General guidelines on ways to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available on the EPA Ground Water and Drinking Water Website at https://www.epa.gov/ground-water-and-drinking-water.

What does this mean?

This is not an emergency. If it had been, you would have been notified within 24 hours.

*Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites which can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.*

These symptoms, however, are not caused only by organisms in drinking water, but also by other factors. If you experience any of these symptoms and they persist, you may want to seek medical advice.

What is being done?

[Describe corrective action.] We anticipate resolving the problem within [estimated timeframe] (or the problem was resolved on [give date]).

*For more information, please call [name of water system contact] of [water system name] at [phone number].*

*Please share this information with all the other people who drink this water, especially those who may not have received this notice directly (for example, people in apartments, nursing homes, schools, and businesses). You can do this by posting this notice in a public place or distributing copies by hand or mail.*

This notice is being sent to you by [water system name]. State Water System ID# _______.

Date distributed: _______